Here's a step-by-step explanation of how the upgraded Smart Room Light Controller circuit works, from beginning to end, using the 4 logic ICs (7404, 7408, 7432, 7486) and all components:

🧠 Objective Recap:

Turn ON the room light only when:

- It’s dark (based on LDR),

- Motion is detected (via PIR sensor),

- Energy-saving mode is OFF or the user manually overrides the system.

⚙️ 1. LDR + NOT Gate (Darkness Detection)

- Component Used: LDR (Light Dependent Resistor) + 7404 (NOT Gate).

- How It Works:

  - In bright light, LDR gives LOW voltage → after the NOT gate = HIGH → meaning NOT dark.

  - In dark, LDR gives HIGH voltage → after NOT gate = LOW → meaning DARK condition is TRUE.

- ✅ Output of this NOT gate represents “Is it dark?”

🕵️‍♂️ 2. PIR Sensor (Motion Detection)

- Component Used: PIR Motion Sensor.

- How It Works:

  - When motion is detected, PIR gives a HIGH signal.

  - When no motion, output is LOW.

- ✅ This is one of the main inputs for automation.

🔀 3. Energy-Saving Mode – SPST Toggle Switch + XOR Gate

- Component Used: Toggle Switch (SPST) + 7486 (XOR Gate).

- How It Works:

  - SPST Switch represents Energy-Saving Mode (ON/OFF).

  - XOR gate compares:

    - Motion detection (from PIR)

    - Toggle switch state

  - XOR logic:

    - If Energy-Saving = OFF (0) and Motion = ON (1) → Output = 1

    - If Energy-Saving = ON (1) and Motion = ON (1) → Output = 0 (light won’t turn on)

- ✅ This allows the system to save energy even if there’s motion.

🤝 4. Combine DARK + (Motion XOR Mode) – AND Gate

- Component Used: 7408 (AND Gate).

- Inputs:

  - Output of NOT gate (Darkness condition)

  - Output of XOR gate (Motion + Energy-saving condition)

- How It Works:

  - If it’s dark AND there is valid motion input, then output is HIGH.

- ✅ This is the Automated Control Output.

👆 5. Manual Override – Pushbutton

- Component Used: Simple Pushbutton (normally open).

- How It Works:

  - When pressed, gives HIGH signal.

  - This lets the user turn the light on manually, regardless of motion or darkness.

➕ 6. Final OR Gate (Automatic + Manual Control)

- Component Used: 7432 (OR Gate).

- Inputs:

  - AND gate output (auto mode).

  - Pushbutton output (manual override).

- How It Works:

  - If any one is HIGH → OR gate output is HIGH → Light turns ON.

- ✅ This is the Final Control Signal to LED Light.

💡 7. Output to LED Light

- Component Used: LED (with resistor).

- Connected to: Output of OR gate.

- Light Turns ON When:

  - Auto condition is met OR

  - Manual button is pressed.

⚡ Final Flow Summary:

| Input | Condition | Result |
| --- | --- | --- |
| LDR says “It’s dark” | TRUE (via NOT gate) | Proceed check |
| PIR detects motion | TRUE | Proceed check |
| Energy-saving mode OFF | TRUE (via XOR) | Proceed check |
| Manual button pressed | TRUE | Immediate light ON |
| Final output (OR gate) | Any TRUE input | LED turns ON |

Let me know if you want a diagram or animation of this logic flow too!